Practical 6

Aim: Write a VHDL Code to Implement 4- bit up-down counter using JK flip flop.... Code: library IEEE; use IEEE.STD_LOGIC_1164.ALL; entity U_D_Count_4b is Port ( M : in STD_LOGIC; Mb : inout STD_LOGIC; clk : in STD_LOGIC; rst : in STD_LOGIC; cout : out STD_LOGIC_VECTOR (0 to 3)); end U_D_Count_4b; architecture Behavioral of U_D_Count_4b is component JK_FF is Port ( clk : in STD_LOGIC; rst : in STD_LOGIC; J : in STD_LOGIC; K : in STD_LOGIC; Q : out STD_LOGIC; Qb : out STD_LOGIC); end component JK_FF; component and_gate is Port ( a : in STD_LOGIC; b : in STD_LOGIC; y : out STD_LOGIC); end component and_gate; component or_gate is Port ( a : in STD_LOGIC; b : in STD_LOGIC; y : out STD_LOGIC); end component or_gate; signal cout1:std_logic_vector(0 to 3); signal q1:std_logic_vector(0 to 3):="0000"; signal d:std_logic_vector(0 to 2); signal u:std_logic_vector(0 to 2); signal c1:std_logic_vector(0 to 2); --signal Mb:std_logic; begin Mb <= not M; z1:for i in 0 to 3 generate f1: if (i=0)generate x1:JK_FF port map(clk,rst,'1','1',cout1(i),q1(i)); end generate f1; f2: if (i>0) generate x2:and_gate port map(cout1(i-1),Mb,d(i-1)); x3:and_gate port map(q1(i-1),M,u(i-1)); x4:or_gate port map(d(i-1),u(i-1),c1(i-1)); x1:JK_FF port map(c1(i-1),rst,'1','1',cout1(i),q1(i)); end generate f2; end generate z1; cout<=cout1; end Behavioral;

RTL DIAGRAM:

Test bench Code :

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity Tb_U_D_Count_4b is

--  Port ( );

end Tb_U_D_Count_4b;

architecture Behavioral of Tb_U_D_Count_4b is

component U_D_Count_4b is

Port ( M : in STD_LOGIC;

Mb : inout STD_LOGIC;

clk : in STD_LOGIC;

rst : in STD_LOGIC;

cout : out STD_LOGIC_VECTOR (0 to 3));

end component U_D_Count_4b;

signal m1,mb1,clk1,rst1:std_logic;

signal cout1:std_logic_vector(0 to 3);

begin

X1:U_D_Count_4b port map(m1,mb1,clk1,rst1,cout1);

process

begin

rst1<='0';

wait for 5ns;

rst1<='1';

wait;

end process;

process

begin

clk1<='0';

wait for 5ns;

clk1<='1';

wait for 5ns;

end process;

process

begin

m1<='1';--upcount

wait for 500ns;

m1<='0';--downcount

wait;

end process;

end Behavioral;

SIMULATION WAVEFORM :

SYNTHESIS SUMMARY:

Maximum Combinational Delay: 5.351nSec

Aim: Write a VHDL Code to implement 4-bit serial in-parallel out shift register.... Code: library IEEE; use IEEE.STD_LOGIC_1164.ALL; entity Sin_Pout_4b is Port ( clk : in STD_LOGIC; rst : in STD_LOGIC; I : in STD_LOGIC; Q : out STD_LOGIC_VECTOR (0 to 3)); end Sin_Pout_4b; architecture Behavioral of Sin_Pout_4b is component D_FF is Port ( clk : in STD_LOGIC; rst : in STD_LOGIC; D : in STD_LOGIC; Q : out STD_LOGIC; Qb : out STD_LOGIC); end component D_FF; signal u1:std_logic_vector(0 to 3); signal u2:std_logic_vector(0 to 3); begin z1: for k in 0 to 3 generate f1:if(k=0) generate x1:D_FF port map(clk,rst,I,u1(k),u2(k)); end generate f1; f2:if(k>0) generate x2:D_FF port map(clk,rst,u1(k-1),u1(k),u2(k)); end generate f2; end generate z1; Q<=u1; end Behavioral;

RTL DIAGRAM:

Test bench Code :

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity Tb_Sin_Pout_4b is

--  Port ( );

end Tb_Sin_Pout_4b;

architecture Behavioral of Tb_Sin_Pout_4b is

component Sin_Pout_4b is

Port ( clk : in STD_LOGIC;

rst : in STD_LOGIC;

I : in STD_LOGIC;

Q : inout STD_LOGIC_VECTOR (0 to 3));

end component  Sin_Pout_4b;

signal k,clk1,rst1:std_logic;

signal Q1:std_logic_vector(0 to 3):="0000";

begin

X1:Sin_Pout_4b port map(clk1,rst1,k,Q1);

process

begin

rst1<='0';

wait for 5ns;

rst1<='1';

wait;

end process;

process

begin

clk1<='0';

wait for 5ns;

clk1<='1';

wait for 5ns;

end process;

process

begin

k<='1';

wait for 10ns;

k<='0';

wait for 10ns;

k<='1';

wait for 10ns;

k<='0';

wait;

end process;

end Behavioral;

SIMULATION WAVEFORM :

SYNTHESIS SUMMARY:

Maximum Combinational Delay: 4.076nSec

Aim: Write a VHDL Code to implement 4-bit ripple adder using generate statement..... Code: library IEEE; use IEEE.STD_LOGIC_1164.ALL; entity rpl_adder_4b is Port ( a : in STD_LOGIC_VECTOR (3 downto 0); b : in STD_LOGIC_VECTOR (3 downto 0); c : in STD_LOGIC; s : out STD_LOGIC_VECTOR (3 downto 0); co : out STD_LOGIC); end rpl_adder_4b; architecture Behavioral of rpl_adder_4b is component fulladder is Port ( a : in STD_LOGIC; b : in STD_LOGIC; c : in STD_LOGIC; s : out STD_LOGIC; cout : out STD_LOGIC); end component fulladder; signal c1:std_logic_vector(0 to 2); begin z1: for i in 0 to 3 generate f1:if(i=0) generate x1:fulladder port map(a(i),b(i),c,s(i),c1(0)); end generate f1; f2:if(i=3) generate x2:fulladder port map(a(i),b(i),c1(i-1),s(i),co); end generate f2; f3:if(0<i and i<3) generate x3:fulladder port map(a(i),b(i),c1(i-1),s(i),c1(i)); end generate f3; end generate z1; end Behavioral;

RTL DIAGRAM:

Test bench Code :

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity Tb_rpl_adder_4b is

--  Port ( );

end Tb_rpl_adder_4b;

architecture Behavioral of Tb_rpl_adder_4b is

component rpl_adder_4b is

Port ( a : in STD_LOGIC_VECTOR (3 downto 0);

b : in STD_LOGIC_VECTOR (3 downto 0);

c : in STD_LOGIC;

s : out STD_LOGIC_VECTOR (3 downto 0);

co : out STD_LOGIC);

end component rpl_adder_4b;

signal a1 : STD_LOGIC_VECTOR (3 downto 0);

signal b1 : STD_LOGIC_VECTOR (3 downto 0);

signal c1 : STD_LOGIC;

signal s1 : STD_LOGIC_VECTOR (3 downto 0);

signal c01 : STD_LOGIC;

begin

X1:rpl_adder_4b port map(a1,b1,c1,s1,c01);

process

begin

c1<='0';

a1<="0000";

b1<="0000";

wait for 10ns;

a1<="1111";

b1<="0000";

wait for 10ns;

a1<="1111";

b1<="1111";

wait for 10ns;

a1<="1111";

b1<="1010";

wait for 10ns;

c1<='1';

a1<="0000";

b1<="0000";

wait for 10ns;

a1<="1111";

b1<="0000";

wait for 10ns;

a1<="1111";

b1<="1111";

wait for 10ns;

a1<="1111";

b1<="1010";

wait for 10ns;

end process;

end Behavioral;

SIMULATION WAVEFORM :

SYNTHESIS SUMMARY:

Maximum Combinational Delay: 5.942nSec